- Models and Related Technology

We describe the preliminaries of SWIB, including network model, blockchain setting, communication model, and attack models. When designing a blockchain consensus protocol, we first require to determine a network model that is the fundamental infrastructure running the protocol. Then, we should consider the data architecture of distributed ledger used in our protocol. To improve the performance of blockchain protocol, we use a threshold BLS signature scheme as voting mechanism to reduce consensus latency. Since computing time is negligible small, we can analyze the performance of consensus protocol by giving a communication mode. In order to discuss the attacks-resistance of consensus protocol, attack models should be predefined.

3.1 Network Model

Blockchain consensus protocols are designed according to network models. Consensus nodes achieve consensus through networks transmitting messages. We only consider a wireless broadcast network consisting of nodes, which are located within communication range of each other, and communicating with each other by transmitting messages. All nodes have same functions. Each node equipped with transceiver works in a half-duplex manner. This means that nodes can transmit or receive messages, but not both simultaneously. In practice, such a network can be formed by a group of unmanned aerial vehicles or intelligent vehicles.

Nodes adopting digital signature technology can achieve node identity confirmation and the verification and integrity of communication messages. In digital signature, each node has its key pair, which used to message encryption and decryption, and generate signature. We assume that each node can get its private-public key pair and a main public key by independently running a secure distributed key generation protocol. Each node can obtain the public keys and identities of other nodes by exchanging messages. Thus, each node knows the identities and public keys of all other nodes.

3.2 Blockchain

Blockchain is a public electronic ledger, which similar to a relational database. This ledger is openly and simultaneously shared among various nodes in system. Blockchain uses a consensus protocol to ensure the dependability and integrity of blockchain system. Therefore, blockchain can only be updated when consensus is reached on a new block between nodes in the system. Each node maintains a local blockchain, which is composed of blocks chronologically linked to create a chain. Each block contains a block header and a block body. The block header records blockchain version, block proposer, block height, previous hash, block hash, block full signature, and transactions hash root, etc. The block body stores transaction recodes, which consist of payer's information, payee's information and other necessary contents. Figure 1 shows the data structure of a blockchain.

Fig. 1 Blockchain

3.3 Threshold BLS Signature Scheme Technology

Threshold Boneh-Lynn-Shacham (BLS) signature scheme [29] is the combination of threshold signature scheme [34] and BLS signature scheme [30]. The signature share generation of threshold BLS signature scheme concurs with the BLS, and the aggregation of full signature is congruent with threshold signature scheme. BLS signature scheme is closely related to bilinear maps and Gap Diffie-Hellman (GDH) groups, where the Computational Diffie-Hellman problem is hard but the Decision Diffie-Hellman problem is easy. Threshold signature scheme allows multiple participants securely reconstruct a secret (i.e., a main private key) and perform computation (i.e., signature generation or decryption) even an adversary has corrupted some participants. Threshold BLS signature scheme is secure, which means that the output of the scheme is unforgeable and robust.

One important property of the scheme is uniqueness, which means that any set of more than t=N+12 partial signature shares can recover the full signature. Another important property is verifiability, which means that the full signature can be verified by anyone using the unique main public key.

Threshold BLS signature scheme includes key generation algorithm, signature generation algorithm and verification algorithm. The key generation algorithm adopts a discrete log-based distributed key generation method [31] to distribute private-public key pair and a main public key to participants. In threshold BLS signature scheme, participants jointly sign a message through the signature generation algorithm. The full signature of the message can be obtained when a sufficient number of participants sign the message. All participants can use the main public key to verify the full signature by the verification algorithm.

Threshold Boneh-Lynn-Shacham (BLS) signature scheme [29] can be used to help consensus protocols improve the performance of system. Consensus protocols can adopt the threshold BLS signature scheme as a voting mechanism to improve the efficiency of consensus. All consensus nodes use a share of a main private key as their private keys. In a consensus process, each node generates a partial signature of a proposal as the vote of the node for the proposal. The full signature of the proposal can be recovered when nodes aggregate a sufficient number of partial signatures. Since several partial signatures can be aggregated into a full signature, nodes can verify multiple votes in an operation. In addition, the termination of a consensus process not relies on the leader of system. Any consensus node can terminate a consensus process when it constructs the full signature and broadcasts the full signature to others. Therefore, threshold BLS signature scheme can help consensus nodes quickly and steadily achieve consensus. In SWIB, we use threshold BLS signature scheme as mechanism to improve the performance of blockchain system. To ensure the security of SWIB, we assume that the number of honest nodes should satisfy the requirement of threshold BLS signature scheme.

3.4 Communication Model

In SWIB, we mainly focus on the impact of wireless communication in the performance of blockchain system. As a metric that used to evaluate the performance of blockchain system, consensus latency consists of computing time and communication time. However, the computation load of nodes in SWIB is small, which means the computing time is negligibly small. During consensus process, nodes broadcast messages over wireless channel to achieve consensus. We consider a communication model with p-persistent carrier-sense multiple access (CSMA). Nodes continuously sense channel and transmit message with a constant probability when detecting channel idle. We assume that wireless channels follow the Rayleigh fading model [32]. The channel gain between nodes follows the complex normal distribution. According to the Rayleigh fading model, the received signal-to-noise (SNR) ratio over the channel between nodes can be computed.

During consensus process, messages losses can lead to consensus failure. Message losses are mainly caused by channel collision and channel fading. Therefore, a successful transmission should satisfy two conditions: 1) if and only if there only one node transmitting in a time slot; and 2) the SNR ratio is equal to or bigger than the target one. In channel contention process, nodes compete for the channel with a constant transmit probability. Only if one node transmits in a mini time interval can the node transmit message successfully. Even a node competes successfully, it may fail to transmit a message due to channel fading. The SNR ratio over channel between nodes varies with the time-variant communication environment. When SNR ratio is less than a given target threshold, the communication between nodes is interrupted. In order guarantee communication under unreliable networks, retransmission mechanism is necessary. When communication interruption occurs, retransmissions are carried out until the lost messages are successfully delivered.

3.5 Attack Models

Attacks launching by adversary can halt consensus process. The attack-resistance is very important for security of blockchain system. We assume that an arbitrary adversary can control no more than 50% of the total voting power and corrupt nodes. In SWIB, we only consider two kinds of attacks of adversary: Sybil attack and jamming attack.

- Adversary can launch Sybil attack, in which adversary can generate a large number of identities for corrupted nodes. These nodes can deviate from consensus protocol. Adversary may obtain inappropriate power in consensus process. Adversary can overwhelm other nodes through majority voting power, then control the generation of blocks and prevent other new nodes from entering system.

- Adversary can launch jamming attack to interfere with the message transmission. Jamming attack is a denial-of-service attack that adversary prevents other nodes from using the channel to communicate by occupying the channel they are communicating on. Consensus process can be interrupted when adversary launches jamming attack. Adversary can prevent other nodes from broadcasting their votes. A consensus process cannot be complete if the number of votes is less than the secure threshold of blockchain system. In Blockchain system adopting communication-based consensus protocols will loss liveness when consensus process is interrupted. Adversary can prevent other new nodes from entering system by jamming the requests of these new nodes. Therefore, the security of the system will be halted.

We design SWIB that can resistant both Sybil attack and jamming attack. Even attacked by adversary, blockchain system can still work securely.

- The Stable-aware Wireless Blockchain Consensus Protocol

In this section, we propose a stable-aware wireless blockchain consensus protocol, abbreviated as SWIB. We first present an overview of the protocol, and then give the details of SWIB. To ensure the security of the protocol, we propose incentive and publishment mechanisms and a synchronization mechanism.

4.1 Overview

In this subsection, we present the overview of SWIB, which adopts a four-stages framework to achieve consensus in blockchain system round-by-round (As shown in Fig. 2). The four stages of SWIB include block proposer election stage, block generation stage, block verification stage, and block finalization stage. SWIB adopts a random block proposer election algorithm and a threshold BLS signature scheme to prevent inherent forks and reduce system overhead. The random block proposer election algorithm used in block proposer stage can reduce adversary corruption risk and prevent inherent forks. The threshold BLS signature scheme used in both block verification and finalization stages can reduce the system overhead of consensus process.

Fig. 2 An overview of SWIB in a round r. At the beginning of the round, all consensus nodes are assumed to maintain a same replica of the blockchain. (1) Block proposer election algorithm is executed to randomly determine the block proposer for the current round; (2) the elected block proposer will generate a new block, and broadcast to other nodes; (3) verify legality of the new block, and run signature generation algorithm to vote for valid block; (4) run signature aggregation algorithm and signature recovery algorithm to finalize the block when generating the full signature.

In SWIB, each round consists of the four stages, whose detailed description will be given in the following. In the block proposer election stage, a block proposer will be randomly elected from all consensus nodes via random block proposer election algorithm. This algorithm is based on the roulette wheel selection algorithm and uses both remaining active time and the number of generated blocks as the elected weight of nodes. In the block generation stage, the elected block proposer will generate a new block, which contains a set of transactions and relevant information of previous blocks and block proposer. The block will be broadcast to all other consensus nodes in the blockchain system. SWIB uses a secure threshold BLS signature scheme as voting mechanism, which is related to both block verification stage and finalization stage. Upon receiving the block, nodes start block verification stage. In this stage, nodes will generate a partial signature of the block as the vote of the block if the legality of block proposer and the content of the block is verified. In block finalization stage, the block will be finalized when it is approved by enough nodes. Node can aggregate a sufficient number of partial signature shares into a full signature, which can be seen as the proof of block finalization. Generating a full signature indicates that a sufficient number of nodes signed the block. Upon receipt or generation of a full signature, nodes will append the block into their local blockchain. After that, nodes will start a new round.

To join a blockchain system, nodes require to obtain identities and system relevant information. New nodes get identities and active time by depositing certain amount of money. These money will be stored in a virtual account, and can only be taken out by the depositor through executing an unpledged operation. Then, nodes obtain their private-public key pair and a main public key via a distributed key generation algorithm. Nodes gain the information of other nodes and blockchain by exchanging messages. Nodes start participating consensus process when they have the latest blockchain and a consensus node list that ordered by the hash value of their public keys.

4.2 The SWIB Protocol

SWIB protocol is a four-stages framework blockchain consensus protocol, which achieves consensus on blocks among consensus nodes. In the following, we shall present the details of these stages.

4.2.1 Block Proposer Election Stage

In block proposer election stage, we present a random block proposer election algorithm based on a roulette wheel selection to randomly elect a unique block proposer. The random block proposer election algorithm (see Algorithm 2) first generates a random value to ensure the security of election process. Then, a roulette wheel will be constructed according to the stability of nodes. The elected probability of nodes in the wheel is proportional to their stability to ensure the fairness of election process. Finally, a block proposer of current round is determined according to the random value and the wheel constructing by elected probability of nodes.

A distributed randomness generation function allows consensus nodes to independently produce an unpredictable and verifiable random value. In each round, the calculation of the random value is performed as

Rdmr=Hr,BHr-1,sigfullr-12lenHr,BHr-1,  sigfullr-1,

where r is the current round number, and BHr-1 is the hash of the latest confirmed block, and sigfullr-1 is the full signature that revealed at the end of the previous round. Although the hash value BHr-1 and the round number r are pre-known parameters, the full signature sigfullr-1 can only be revealed at the end of round r-1. Thus, it is impossible for nodes to predict the random value in advance. The full signature can be verified through the signature verification algorithm of threshold BLS signature scheme. Since all nodes have same inputs, the random value of nodes should be consistent and verifiable. Besides, even a node first recovers full signature before others, the full signature cannot be tampered due to the security of threshold BLS signature scheme. (要在前面提一下安全性，查一下所有用the的地方) Both unpredictable and verifiable(名词) of random value ensure the security of block proposer election.

The stability is determined by active time and the number of generated blocks in latest rounds. We define the active time ratio of a node as the proportion of the active time of the node in the cumulative active time of all consensus nodes in system. And we define consensus ratio of a node as the proportion of the number generated blocks of the node in a fixed number of the latest blocks on blockchain. According to the two definitions, we can give the expressed formula of stability(签名介绍active time…)

Sv=a×Ta∑Ta+b×NvK

where ρv=Ta∑Ta is the active time ratio, and rv=NvK is the consensus ratio, a and b (a+b=1, a≥0, b≥0) are the weights of active time ratio and consensus ratio, respectively.

Utilizing the stability of nodes as weight can ensure electing a high-quality block proposer and the fairness of the election process. (展开说明是如何确保的？)

The block proposer election algorithm is based on the roulette wheel selection. A roulette wheel is constructed from the node elected probability, which is defined as the ratio of the stability of a node and total stability of all nodes. The wheel is represented in the form of a pie chart, where the area occupied by each node on the roulette wheel is proportional to its elected probability. According to the elected probabilities of nodes, we can calculate the cumulative elected probabilities of nodes. In the algorithm, a random value in the interval [0, 1) is generated via a distributed randomness generation function. When the cumulative elected probability is greater than the random number, the corresponding node is elected as current block proposer. Since a node with better stability value will occupy a bigger area in the pie chart, the node will have high  probability being elected.

4.2.2 Block Generation Stage

A block will be generated in block generation stage (see Algorithm 3). In general, the elected block proposer will generate a new block, which contains transaction records and other necessary information. Then, the block proposer will broadcast the new block to other nodes. When receiving the block successfully, nodes will immediately start block verification stage.

A block contains a header and a body. The header consists of the round number, the hash value of previous block, the current block hash, the identity of the block proposer, the full signature and the root of transactions, and so on. Block body usually stores transaction recodes. When generating a block, most of these data are packaged into the block. Specially, the full signature can only be appended to this block in block finalization stage.(all…but把前面两句合成一句)

4.2.3 Block Verification Stage

In block verification stage, all nodes will verify the new block from current block proposer. When receiving a block, consensus nodes will verify the legality of block proposer and the content of the block. Nodes will vote for the current block.

In block verification stage, a node needs to check the following conditions:

- Block proposer: The output of block proposer election algorithm is the same with the identity of block proposer.

- Previous hash: The previous hash value in the current block is equal to the hash of last block on its blockchain.

- Transactions: All transactions included in the current block should not conflict with previous confirmed transactions.

If all the above conditions are satisfied, then each node generates partial signature for the block as a vote. Nodes will broadcast their votes to other nodes. The pseudo code of block verification is presented in Algorithm 4.

4.2.4 Block Finalization Stage(Full Signanature 也有一个验证，放在上一个阶段)

In block finalization stage, a block will be finalized. Nodes will vote for the current block. If there are a sufficient number of nodes agree on the current block, nodes will be informed to finalize the block. Each node will update the block in their local blockchain. Then, nodes will start a new consensus round. The pseudo code of block finalization is presented in Algorithm 5.（看不出来finalized 是在干嘛）

A full signature is used as a(the-所述) proof of the finalization of blocks in the block finalization stage. SWIB utilizes a voting mechanism that based on a secure threshold BLS signature scheme. In this voting mechanism, a partial signature is seen as a vote of the proposed block. When gathering a sufficient number of partial signature shares of the proposed block, a consensus node can recover the full signature through the signature recovery algorithm of the threshold BLS signature scheme. The successful reconstruction of full signature indicates that a sufficient number of nodes approve the proposed block. In this case, the block is considered to be valid. Therefore, it is feasible to use the full signature as the proof of block finalization.

4. 3 Incentive and Punishment Mechanisms

In this subsection, we present incentive and punishment mechanisms to improve the security of SWIB. On the one hand, an incentive mechanism is necessary in a blockchain system to encourage consensus nodes in participating the consensus process. On the other hand, a punishment mechanism is used to reduce the risk that rational nodes become malicious.

An incentive mechanism can improve the security of SWIB. Nodes might be reluctant in participating consensus process due to considerable computational power consumption and transmission power consumption. In consensus process, rational nodes may be willing to wait for receiving full signature from others, rather than consuming computational power to verify a block, generate partial signature, and recover full signature. Reward can motivate nodes to participate the consensus process actively. This incentive mechanism encourages nodes to generate valid block and signatures, which are necessary for consensus process. Consensus nodes would like to follow the SWIB to obtain reward. Therefore, the security of consensus process will be improved through the incentive mechanism.

Our incentive mechanism based on a secure threshold BLS signature scheme will reward those nodes that have contribute to consensus process. The reward in blockchain system are the submitted transaction fees in blocks. Part of the transaction fees is distributed to block proposers, and the rest fee will be shared among those nodes whose partial signatures are used to recover the full signature of a block. Specially, there are more than one set of partial signature shares recover the full signature at same time. In this case, the incentive mechanism will distribute reward to those nodes, whose partial signature shares that aggregate into the full signature have the smallest average timestamp.

A punishment mechanism can improve the security of blockchain system. Nodes can deviate from SWIB to harm consensus process. Adopting a punishment mechanism can suppress such malicious behaviors from harming blockchain system continuously. The punishment mechanism will punish malicious nodes by taking back reward, reducing the probability of obtaining reward, or depriving the right to participant in consensus process. Nodes would more likely to follow system rule under the punish mechanism. As a result, the security of blockchain system will be improved.

Punishment mechanism suppresses malicious behaviors by reducing the active time of a node. The active time of a node is related to its stability, which is proportional to the probability that electing the node as block proposer. Reducing the active time of a malicious node can decrease the elected probability of the node. Besides, the active time of nodes determines how long the nodes can participate consensus in a blockchain system. Malicious nodes will be quickly expelled from the system if they behave malicious frequently. Therefore, it is feasible to use reducing active time as a punish way in the punishment mechanism.

4. 4 Synchronization Mechanism

Synchronization mechanism is important to guarantee the security of consensus process. If consensus nodes have distinct view on blockchain, the previous hash of the processing block may be different with the hash of the latest block on their blockchain. Since the result of block verification is false, these nodes will not vote for the block. As result, consensus may be not achieved among these nodes. The synchronization mechanism can ensure that nodes have the latest blockchain all the time. Therefore, consensus process will not be interrupted in block verification due to inconsistent blockchain.

Our synchronization mechanism improves the security of synchronization process by allowing nodes request blocks from multiple high-quality nodes. In SWIB, nodes with higher stability are more likely to maintain the latest blockchain, and have lower risk of sending faulty blockchain information. The synchronization mechanism allows new nodes to request a part of blockchain information from multiple high-quality nodes. This will reduce the risk that a node transmits error blockchain information to mislead new nodes. Once a node transmits error information, the receiving node can immediately detect it through blockchain information received from other nodes. The new node can request the of blockchain information from other trustworthy nodes to ensure the receipt of correct blockchain information.

The synchronization mechanism can reduce the burden of nodes in synchronization process. It is a huge burden for a node to transmit a large number of blocks. The node consumes large transmitting power and transmitting time to complete this transmission. Actually, all consensus nodes have obligation to response for the request of blockchain information. In our mechanism, nodes are allowed to request a part of blockchain information from multiple nodes. For the requested nodes, the transmit power consumption Thus, this mechanism decreases the burden of a single requested node.(缺乏算法伪代码)

- Protocol Analysis

In this section, we will analyze the performance and security of SWIB. The performance is analyzed in terms of consensus success probability and consensus latency. The security is analyzed in terms of consensus security, random generation security, and attack resistance.(安全性分析放在性能分析的前面)

5.1 Performance

We analyze consensus success probability and consensus latency, which is important metrics to measure the performance of a blockchain consensus protocol.

Since all messages of running SWIB are transmitted over wireless networks, we analyze the performance of our protocol by a wireless communication model. Wireless channels in our consider communication model follow the standard path-loss model and the Rayleigh fading model. In detail, the path-loss of the wireless channel between nodes u and v is modeled as

PLdBdu,v= PLdBd0+10log10du,vd0α,

where PLdBd0 is the path-loss at reference distance d0, and du,v is the distance between nodes  u and v, and α∈(2, 6] is path-loss exponent. Let Hu,v be the channel gain from u to v, following the complex normal distribution with zero mean and variance Pt (i.e., Hu,v∼CN(0, Pt)). When a signal is transmitted from u to v with transmission power Pt, the Signal-to-Noise (SNR) ratio at receiving node v is formulated as

SNRuv(Hu,v)=10- PLdBd010⋅  Pt⋅Hu,v2Pn⋅du,vd0-α

wherePt is transmit power, and Pn is the additive white Gaussian noise.

Since the time in our protocol implementation is divided into synchronized slots, in which can complete a packet transmission. The fixed time interval of a slot is calculated as follows:

τo= mB⋅log(1+β)

where m is the maximum size of packet sent by nodes during the consensus process in bits, and β is the target SNR ratio of transmission, and B is the bandwidth utilized for transmission. Since a message will be divided into multiple packets in transmission process, the time interval of transmit a message should be τ=l⋅τo, where l is the number of packets of the message.

In our communication model, each node competes for accessing the channel to broadcast messages over wireless channels. In channel contention process, the transmission between nodes will fail if more than one node transmits in a same slot. Channel contention success probability can be expressed as

pcomp(N)=Np1-pN-11-1-pN

where N is the number of consensus nodes, and p is a constant transmit probability of each node.

During transmission process, the SNR ratio between nodes varies with the time-variant communication environment. When SNR ratio is less than a given target threshold, the communication between nodes is interrupted. Since Hu,v2 is exponentially distributed, the SNR outrage probability between nodes u and v is expressed as follows

ϵu,v(Hu,v)=PrSINRu,v<β= Pr|Hu,v​2<10- PLdBd010⋅βPnPt⋅du,vd0α=1-e-10 PLdBd010⋅βPnPt⋅du,vd0α.

Since nodes compete for accessing the channel, each node is allocated a contention time duration for channel compete process. For a given target contention success probability, the minimum time slots of contention process can be computed. We have the following proposition.

Proposition 1. We assume that the total number of nodes in system is N. Each node competes for the channel with a constant probability p. For a given target contention success probability 0≤ς<1, the required time slot of contention process is

xcomp=log1-ςlog1- pcomp

where pcomp=Np1-pN-11-1-pN is contention success probability.

Proof: In a blockchain system, the total number of nodes is N, and the channel access probability of each node is p. Then, the channel contention success probability should be pcomp=Np1-pN-11-1-pN. Let X be a random variable denoting the number of time slots required for a node compete successfully. We suppose that xcomp is the minimum value such that

PrX≤xcomp=1-1-pcompx≥ς

for a target contention success probability 0≤ς<1. Then, the required number of time slots is calculated as

xcomp=log1-ςlog1-pcomp.

In our consider models, each transmitting node is allocated a broadcast time duration for a message broadcast. Each node has its target communication nodes. For a given target broadcast success probability, the minimum time slots of a node required for packet transmission to all target communication nodes can be computed. We can obtain the proposition.

Proposition 2. For a given probability 0≤ξ<1, the required number of time slot to broadcast a packet from a node to its target nodes is

xtrans=log1-ξ1nlogϵmax

where n is the number of the target nodes, and ϵmax= 1-e-10 PLdBd010⋅βPnPt⋅dmaxd0αis the SNR outage probability between the node and its maximum distance target node.

Proof: Let X be a random variable denoting the number of time slots require to transmit a message from node v to node u via broadcast. Since transmission failure might occurs with a certain probability due to channel fading, the message will be sent repeatedly until successful transmit. Therefore, the probability density function of the random variable X is represented as

fX= ϵv,uX-11-ϵv,u,  X≥1,0,  otherwise

where ϵu,v is the SNR outage probability between the two nodes. Then, the broadcast outage probability bounded as

PrmaxuXu≤x=u∈V∖{v}Pr[Xu≤x]=u∈V∖{v}1-ϵv,ux≥1- ϵmaxxn

where ϵmax is the SNR outage probability between the node and its maximum distance target node. Since the target broadcast success probability ξ is given, we can compute the required time slots as

xtrans=log1-ξ1nlogϵmax.

5.1.1 Consensus Success Probability

The performance of SWIB is analyzed according to the consensus success probability under wireless networks with unstable and unreliable channel in this subsection.

Consensus will fail if no node recovers the full signature in the consensus round. In our considered models, we have the following consensus success probability of SWIB.

Proposition 3. For a given round, the consensus success probability of our protocol is lower bounded as

pCons=1-f=N-12N-1N-1f1-ptransf⋅ptransN-1-f N

where the N is the number of nodes, ptrans= Np1-pN-11-1-pN(1-ϵmax) is the transmission success probability, and ϵmax= 1-e-10 PLdBd010⋅βPnPt⋅dmaxd0α is the maximum SNR outage probabilities.

Proof. A node can transmit a message successful when it competes for accessing channel successfully and transmits the message over wireless channel successfully. Let pcomp be the transmission contention successful probability, and ϵ be the SNR outage probability between nodes. Then, the transmission success probability of a node is computed as

ptrans=pcomp⋅(1-ϵ)

Aggregation failure occurs when a node fails to recover a full signature by aggregating more than N-12 partial signature from other nodes in consensus process. Let V be the set of consensus nodes in a round. For any node v∈V, the aggregated failure probability of the node due to transmission failure is given as

pAggr= f=N-12N-1N-1f1-ptransf⋅ptransN-1-f

If all consensus nodes failed to recover the full signature, the consensus process in a round will be interrupted. The consensus success probability is calculated as

pCons=1-v∈Vp Aggr

=1-v∈Vf=N-12N-1N-1f1-ptransf⋅ptransN-1-f

=1-f=N-12N-1N-1f1-ptransf⋅ptransN-1-f N.

5.1.2 Consensus Latency

In SWIB, consensus latency is defined as the time interval from a block proposal to its finalization. We assume that the computing time is negligible as compared with the time for channel contention and message transmission in wireless network.

Consensus latency consists of channel contention latency and message transmission latency. Each node requires xcomp time slots in contention process. The contention latency of should be calculated as follows

Lcomp=τo⋅xcomp

where τois the time interval of a slot. Transmission latency is the time that transmitting all messages in a consensus round. The transmission latency of the SWIB is expressed as

Ltrans=τo⋅TB+ τo⋅i=1N-1Tsig+τo⋅Tsig

where TB=MBm  is the required time slots of transmitting a block, and Tsig=Msigm is required time slots of transmitting a signature. In SWIB, the consensus latency can be computed as follows:

Lcons= τ0⋅[xcomp+TB+N(xcomp+Tsig)]

5.2 Security Analysis

In this subsection, we analyze the security of SWIB in terms of consensus security, the random generation, and attacks resistance.( N-12想办法处理一下)

（我们的论文与PoW（能耗小），PoS（防止寡头）比有什么优势）

5.2.1 Consensus Security

We conduct a security analysis to show that SWIB protocol provides persistence and liveness to wireless blockchain system. With majority of consensus nodes being honest, our protocol satisfies:

- Persistence: If an honest node proclaims block B is valid, then other honest nodes also agree on that the validity of block B.

- Liveness: All honest nodes eventually commit a block, and generate the following round randomness.

Threshold BLS signature scheme is secure, which means that the output of the scheme is unforgeable and robust. One important property of the scheme is uniqueness, which means that any set of more than t=N+12 partial signature shares can recover the full signature. Another important property is verifiability, which means that the full signature can be verified by anyone using the unique main public key.

The security of SWIB relies on a secure threshold BLS signature scheme. In our protocol, consensus nodes can securely proceed to the following round when a full signature is recovered. However, recovering the full signature requires a sufficient number of partial signature shares. Therefore, only if the number of honest nodes satisfies the requirement of threshold BLS signature scheme, can the security of our protocol be guaranteed.

If there are a quorum of t=N+12 nodes fulfilling the requirement of threshold BLS signature scheme, our protocol can guarantee persistence:

Theorem 1. (Persistence) If an honest node v proclaims block B is the ith block in the blockchain, then other honest nodes should report the same result.

Proof. In order to prove the persistence property, we need to show that for any two blockchains BCu and BCv of honest nodes u and v should have same block at the same position. To prove by contradiction, we assume that Bu and Bv are two different blocks at a same position i of BCu and BCv as maintained by nodes u and v, respectively. There are two cases when the assumption holds:

Case 1. Bu and Bv are respectively appended to the position i of blockchains BCu and BCv in the same round. The block finalization depends on a unique full signature, which is recovered by at least N+12 valid partial signature shares. If two different blocks Bu and Bv are finalized in the same round, it indicates that the threshold BLS signature scheme recovers two different full signatures in a consensus round. This conflicts with the uniqueness property of threshold BLS signature scheme, which contradicts our assumption.

Case 2. Bu and Bv are appended to the blockchains BCu and BCv respectively in two different rounds. Assuming that Bu and Bv are respectively appended to the position i of blockchains BCu and BCv at rounds rm and rn (m<n). According to our protocol, block Bu is finalized in the rm round if the number of nodes voting for the block is at least N+12. Since two different blocks cannot be finalized in the same round, at least N+12 nodes agree that Bu is the ith block of their blockchains in round rm. Using contradiction, we assume that Bv≠Bu. Since rm < rn, node v crashes before round rn and recovers in round rn so that block Bv is appended into BCv when node v updates its blockchain. In this case, at least N+12 nodes have a same view on ith block is Bv at the end of round rn. Since there are at least N+12 nodes agreeing on the ith block in round rm is Bu and the network size is N, we have contradiction such that N>N+12+N+12≥N+1. Hence Bv=Bu, which contradicts the assumption Bv≠Bu.

In summary, all honest nodes are queried for a special position block should respond with the same result. If an honest node says block B is valid, then other honest nodes also agree on its validity.∎

SWIB utilizes a synchronization mechanism to ensure the consistency of nodes. This protocol can guarantee liveness under eventual synchrony. For liveness, we need at least N+12 honest nodes to ensure the finalization of consensus process.

Theorem 2. (Liveness) We suppose that there are at most N-12 faulty nodes in blockchain system, SWIB can finalize a block that proposed by a correct node, and obtain the following round random seed.

Proof. The best case is that no failures occur in blockchain system. Our protocol SWIB can finalize a valid block by recovering the full signature, which can produce the round randomness for the following round.

We are required to prove that our protocol can guarantee liveness under the influence of adversary. The behaviors of malicious nodes controlled by adversary include: 1) intentionally not generating a valid block and 2) refusing to vote for the valid block. There are two cases for node failures:

Case 1. Block proposer is malicious. If the malicious block proposer does not generate any block, all honest nodes will commit on an empty block. If the malicious block proposer generates an invalid block, the block will fail to pass the block validation stage. Thus, all honest nodes will refuse to vote for the invalid block and commit an empty block finally. In this case, each honest node will generate partial signature for the empty block. Once the full signature of the block is reconstructed from enough partial signature shares, the next round randomness is generated successfully. Nodes will start new consensus round if they generate or receive the full signature.

Case 2. At most N-12 malicious nodes refuse to vote for a block. When a non-faulty node generates a valid block, at least N+12 honest nodes will generate partial signature to vote for the block. They will broadcast their signature shares to other nodes. Once collecting a sufficient number of partial signature shares, nodes can recover the full signature and broadcast it to other nodes. These honest nodes can finalize the block and start new consensus round by generate the full signature, which is the proof of block finalization and the randomness of the next consensus round.

In summary, if the number of faulty nodes is at most N-12, honest nodes can commit on a block and generate the following round randomness. ∎

5.2.2 Random Generation

Our distributed randomness generation function is based on a robust threshold BLS signature scheme. Existence of minority of malicious nodes cannot hinder the generation of a valid full signature if more than N+12 consensus nodes are honest. The full signature brings the randomness of block proposer election process. With the robust randomness, adversary cannot corrupt the process. Therefore, block proposer election adopting randomness can guarantee the block proposer is elected unpredictably. However, it does not ensure that the elected block proposer works honestly.

Adversary cannot still control the block proposer election stage even it has high elected probability. Let X be a random value denoting the number consecutive block proposer election that controlled by adversary. Suppose an adversary has less than 50% probability to be elected as block proposer per round, the upper bound of the probability that the adversary controls k consecutive block proposer election is PX≥k< 12k. Let 10-γ be the upper bound of the probability. For example,  γ=7, the adversary can at most control 24 consecutive rounds to become block proposer. This indicates that the probability of adversary controlling more than 24 consecutive rounds is less than 10-7, which is negligible. Therefore, our block proposer election algorithm can ensure that adversary will not always be block proposer.

5.2.3 Attack Resistance

In this subsection, we analyze Sybil attacks resistance and Jamming attacks resistance of SWIB.

SWIB can prevent adversary from launching Sybil attack to benefit from block proposer election stage. Adversary launches Sybil attacks by generating pseudonyms. However, the elected probability will not increase due to increase the number of pseudonyms. The elected probability of a node is proportional to its stability. We assume that the stability of an adversary is SA=aρA+brA, and the total stability of the system is S. If the adversary generates m pseudonyms, whose stability values are sA1= aρA1+brA1, ⋯,sAm= aρAm+brAm(k=1mpAk=pA, k=1mrAk=rA), respectively. The sum of elected probability of the m pseudonyms is

k=1msAkS= k=1mSAkS= k=1maρAk+brAkS= ak=1mρAk+bk=1mrAkS= aρA+brAS= SAS.

which indicates that generating multiple pseudonyms will not increase the elected probability. As a result, our protocol can resistant Sybil attack in block proposer election.

SWIB can ensure the security of block verification stage and block finalization stage when adversary launches Sybil attack. Nodes controlling by adversary will not send messages or send some error messages during consensus process. Once a malicious node becomes block proposer, it may generate an invalid block or does not generate any block. Honest nodes will commit an empty block and finalize it eventually. Since the number of malicious nodes is bounded by N-12, the remaining honest nodes can complete block finalization by aggregating a sufficient number of partial signatures into the full signature. Therefore, our protocol is resistant to Sybil attack as long as adversary controls less than allowed number of voting power.

SWIB can normally operate when adversary launches jamming attacks. Adversary can affect consensus process by jamming the message transmission of honest nodes. If all nodes cannot gather enough partial signature shares to recover the full signature, the liveness of blockchain system cannot be guaranteed. Each node requires to maintain an estimate of adversary time window through sensing the wireless channel to decrease the channel contention under jamming attack. According to the estimate, each node can dynamically adjust their compete probability to ensure the collection of partial signature shares. Besides, system will adjust the timeout duration to guarantee the termination of consensus process according to the channel situation. Therefore, SWIB can ensure the liveness of blockchain system under jamming attacks, further ensuring the security of blockchain system.

In SWIB, utilizing an estimate of adversary time window ensures the finalization of block in wireless blockchain network under jamming attack. We assume that the capability of adversary is (1-δ, T)- bounded, i.e., in any interval of T consecutive slots, adversary can jam no more than (1- δ) T slots, where T∈N and 0< δ ≤ 1. Blockchain system requires to adjust timeout duration when adversary launches jamming attack. Thus, we can obtain the consensus latency under jamming attack.

Proposition 4. Let TC be the number of time slots for each completed consensus round. SWIB requires at least Ljamm= τo⋅N+1⋅T+Tcompδ+TB⋅T+Ttransδ+N⋅Tsig⋅T+Ttransδseconds to against any 1-δ, T-bounded adversary.

Proof. Let Tcomp be the required time slot of channel contention, and Ttrans be the required time slots of a packet transmission. Assuming that in any interval of T consecutive slots, adversary can jam no more than (1- δ) T slots, where T∈N and 0< δ ≤ 1. Thus, a node can transmit a packet successfully after at least (T+Tcompδ)+(T+Ttransδ) time slots. A completed consensus process for each round requires TC=(Tcomp+TB)+N⋅(Tcomp+Tsig) time slots. Therefore, the requires time slots of running our protocol runs under jamming attack is calculated as

F= N+1⋅T+Tcompδ+TB⋅T+Ttransδ+N⋅Tsig⋅T+Ttransδ

where the capability of adversary is (1-δ, T)- bounded. Since the time interval of a slot is τo, the consensus latency under jamming attack should be calculated as

Ljamm= τo⋅N+1⋅T+Tcompδ+TB⋅T+Ttransδ+N⋅Tsig⋅T+Ttransδ.

To reduce the heavy contention under jamming attack, each node maintains an estimate of adversary time window Tv to dynamically adjust their channel compete probability. In this case, SWIB can against (1-δ, T)-bounded adversary launching jamming attacks.